# PD Module 5 — Lab Manual

Powerplan

# Creation of Rails for Standard Cell Placement

- Similar to above, first create pattern, strategy and compile. Since the PG pins of standard cells in the given library are in metal 1, rails are created in M1 only.

icc2_shell > create_pg_std_cell_conn_pattern std_pattern - layers {M1} -rail_width 0.068 -check_std_cell_drc false - mark_as_follow_pin false

icc2_shell > set_pg_strategy rail_start -core -pattern {{name : std_pattern}{nets:{VDD VSS}} } icc2_shell > compile_pg -strategies rail_strat

# Vias b/w Straps and Rails

icc2_shell > create_pg_vias -nets {VDD VSS} -from_layers M1 -to_layers M8 -drc no_check

- Once you have run the above step go to the GUI, zoom into the core area, make sure that you are able to see the metal layers like the below snapshot (fig 5). By the above step we have connected the standard cell rails and made them a part of the power distribution structure.

Fig 5: Standard cell rails connected to the power distribution structure

# Verify PG Nets

- Let’s check for floating wires and vias in the powerplan that we performed.

icc2_shell > check_pg_connectivity

- Now open error browser window (in GUI, Tool bar → View → Error Browser), to view the locations of errors in the GUI.

- Check for DRC errors in the design

icc2_shell > check_pg_drc

- Open error browser window, to view the locations of errors in the GUI.

- Save the Block:

icc2_shell > save_block -as powerplan_done

- Closing the Block and Library:

icc2_shell > close_blocks -force icc2_shell > close_lib icc2_shell > exit