# PD Module 6 — Lab Manual

Placement

# Task

- Open the directory which was created in the previous lab

Linux > cd chiptop2.0 Linux > cd pd/work

- Invoke the icc2 tool

Linux > icc2_shell -o ../logs/placement.log

- Open the Library

icc2_shell > open_lib ../lib/chiptop

- To see all the blocks present in the library

icc2_shell > list_blocks

- Now, open the powerplan_done block which was created in the previous lab

icc2_shell > open_block powerplan_done

- To open the Graphical User Interface (GUI)

icc2_shell > start_gui

- Open one more terminal in the scripts directory, open “placement.tcl” using “vi” or “gvim” editor command from the scripts directory and go through the file contents.

linux > gvim placement.tcl

## Add I/O Buffers

- Now insert buffers for all the IO ports

icc2_shell > add_buffer [get_nets -of [get_ports *]] [get_lib_cells */ SAEDRVT14_BUF_20]

- Do a magnet placement using

icc2_shell > magnet_placement [get_ports *] icc2_shell > legalize_placement -cells [get_flat_cells eco*]

- Fix these buffers

icc2_shell > set_attribute [get_cells eco_cel*] physical_status fixed

Fig 3: I/O Buffer

## Create Placement Blockages

Now, we need to create placement blockages in the channel regions. Select Create → Placement Blockage in the toolbar as shown in fig 4. A window will pop up as shown in fig 5, select whether you need Soft/hard macro and draw the region as shown in fig 6. Placement blockages need to be created for all the channel regions.

Fig 4: Create → Placement Blockage

Fig 5: Select name and type for blockage

Fig 6: Placement Blockage in the channel

## Group Paths

icc2_shell > group_path -name MACRO2REG -from [get_cells -physical_context -filter design_type==macro] -to [all_registers] icc2_shell > group_path -name REG2MACRO -from [all_registers] -to [get_cells -physical_context -filter design_type == macro] icc2_shell > group_path -name INPUTS2REG -from [all_inputs] -to [all_registers] icc2_shell > group_path -name OUTPUTS2REG -from [all_outputs] -to [all_registers] icc2_shell > group_path -name REG2REG -from [all_registers] -to [all_registers]

## Check Design – Pre Placement Stage

icc2_shell > check_design -checks pre_placement_stage

Sample output:

icc2_shell> check_design -checks pre_placement_stage *************************************** Report : check_design Options: { pre_placement_stage } Design : dtmf_recvr_core Version: P-2019.03 Date : Thu May 30 16:07:13 2019 *************************************** Running mega-check 'pre_placement_stage': Running atomic-check 'design_mismatch' Running atomic-check 'scan_chain' Running atomic-check 'mv_design' Running atomic-check 'rp_constraints' Running atomic-check 'timing' Running atomic-check 'hier_pre_placement' *** EMS Message summary *** ------------------------------------------------------------------ Rule Type Count Message ------------------------------------------------------------------ DFT-011 Info 1 The design has no scan chain defined in the scandef. MV-027 Warn 3 The tie-off connection '$tieNet' has not been implemented. MV-208 Warn 1 Pin(s) '$portStr' of library cell '$module' is(are) missing the ... TCK-001 Warn 18 The reported endpoint '$endpoint' is unconstrained. Reason: '$re... ------------------------------------------------------------------ Total 23 EMS messages : 0 errors, 22 warnings, 1 info. ------------------------------------------------------------------

Note: The design is not associated with scan def, set ignore scan def for placement.

icc2_shell > set_app_options -name place.coarse.continue_on_missing_scandef -value true

## Placement Density Setting

- Specify a maximum density that controls how densely the tool can place cells in uncongested areas during wire-length-driven placement

icc2_shell > set_app_options -name place.coarse.max_density -value 0.6

- Specify a maximum utilization that controls how densely the tool can place cells in less congested areas that surround highly congested areas, so that the cells in the congested areas can be spread out to reduce the congestion

icc2_shell > set_app_options -name place.coarse.congestion_driven_max_util -value 0.6

Activity 1:

Bounds are placement constraints given to a particular group of cells to force them to a certain area of the chip.

To create placement bounds run the below commands.

icc2_shell > get_cells -hier * -filter "is_hierarchical == true" icc2_shell > get_cells -hier * -filter "full_name =~ RESULTS_CONV_INST/sub_357_C193*" icc2_shell > create_bound -name bound_1 -boundary {{324.3890 267.4740} {363.1290 304.2580}} -type hard [get_cells -hier * -filter "full_name =~ RESULTS_CONV_INST/sub_357_C193*"]

- To analyze all cells have proper or legal locations

icc2_shell > analyze_lib_cell_placement -lib_cells *

## Perform Coarse Placement

Now, perform coarse placement on the current design, which will place the standard cells in the core area.

icc2_shell > create_placement -congestion

## Set Parasitic Parameters

We had read TLU-plus files during design import stage and named them as best_para and worst_para; here we consider them as early spec and late spec respectively.

icc2_shell > set_parasitic_parameters -early_spec best_para icc2_shell > set_parasitic_parameters -late_spec worst_para

## Check Legality

Once placement is done we look at the different aspects and statistics of the design and check if it has met our expectations or not.

icc2_shell > check_legality -verbose icc2_shell > legalize_placement

Activity 2:

Check the uses of the command legalize_placement

icc2_shell > man legalize_placement

## Timing Report

icc2_shell > report_qor -summary

## Setting VTH Percentage

icc2_shell > set_attribute [get_lib_cells */*LVT*] threshold_voltage_group LVT icc2_shell > set_threshold_voltage_group_type -type low_vt LVT icc2_shell > set_multi_vth_constraint -low_vt_percentage 10 -cost cell_count

## Analyzing Violations That Cannot Be Fixed

icc2_shell > analyze_design_violations

## Perform Placement Optimization

- Once the coarse placement is completed, need to optimize the placed design for timing, electrical DRC violations, area, power, and routability. Then legalize the design placement.

- Run the following command to perform placement optimization.

icc2_shell > place_opt -to final_opto

Fig 8: After Placement

## Check for Global Route Congestion, Cell Density, and Pin Density

Use the Map menu in the GUI to select What-if Channel Congestion, Global Route Congestion, Channel Congestion, Cell Density, Pin Density, and related metrics to visually inspect the placement quality (see Fig. 9).

Fig 9: Congestion / Density Map Mode options

Activity 3:

- Run the command

icc2_shell > report_design

Number of Standard cells : ______________________

Number of macros : ______________________

Number of Buffer/ Inverters : ______________________

Area of Core : ______________________

- Run

icc2_shell > create_utilization_configuration -scope block core_utilization -include {all} icc2_shell > report_utilization -config core_utilization

Utilization Ratio : ______________________

- Run

icc2_shell > report_qor

WNS : ______________________

TNS : ______________________

Number of Violating Paths : ______________________

- Run

icc2_shell > report_timing

Slack : ______________________

## Logically Connect PG Nets with the Pins

- Make the logical connection of PG nets for all the standard cells

icc2_shell > connect_pg_net -net VDD [get_pins -hier * - filter "name == VDD"] icc2_shell > connect_pg_net -net VSS [get_pins -hier * - filter "name == VSS"]

## Saving the Block

icc2_shell > save_block -as placement_done icc2_shell > close_blocks -force icc2_shell > close_lib