# PD Module 7 — Lab Manual

Clock Tree Synthesis (CTS)

# Task

- Open the directory which was created in the previous lab

Linux > cd chiptop2.0 Linux > cd pd/work

- Invoke the icc2 tool

Linux > icc2_shell -o ../logs/cts.log

- Open the Library

icc2_shell > open_lib ../lib/chiptop

- To see all the blocks present in the library

icc2_shell > list_blocks

- Now, open the placement_done block which was created in the previous lab

icc2_shell > open_block placement_done

- To open the Graphical User Interface (GUI)

icc2_shell > start_gui

- Open one more terminal in the scripts directory, open “cts.tcl” using “vi” or “gvim” editor command from the scripts directory and go through the file contents.

linux > gvim cts.tcl

# Check Design – Pre Clock Tree Stage

- Before performing CTS, execute the following command and analyze the report

icc2_shell > check_design -checks pre_clock_tree_stage

Activity 1:

List the clocks and period using the commands.

1. icc2_shell > get_clock or all_clocks

Name of the Clocks used in the design : ______________________

2. icc2_shell > report_clock

Frequency of clock : ______________________

## Define Routing Rule

- Let’s define Non Default Rules for metal 6 and metal 7

icc2_shell > create_routing_rule clk_rule -widths {M6 0.112 M7 0.112 } -spacings { M6 0.112 M7 0.112 }

## Specify Clock Cell List

- The Clock tree is built by using Inverters / Buffers depending on the project requirement.

- Set the choice of buffers that the clock tree must use during CTS.

icc2_shell > set_lib_cell_purpose -include cts [get_lib_cells "saed14rvt_tt0p6v125c/SAEDRVT14_BUF_S_2 saed14rvt_tt0p6v125c/SAEDRVT14_BUF_S_4 saed14rvt_tt0p6v125c/SAEDRVT14_BUF_S_6 saed14rvt_tt0p6v125c/SAEDRVT14_BUF_S_8"]

Activity 2:

Run

icc2_shell > get_lib_cells */*BUF*

Note the different Buffers available in the library.

## Specify Max Fanout

icc2_shell > set_app_options -name cts.common.max_fanout - value 30

## Set Clock Target Skew/Latency

- Similar to Global route congestion, check cell density and pin density maps.

- Set target skew/latency constraints for clock trees

icc2_shell > set_clock_tree_options -clocks [all_clocks ] -target_latency 0.250 -target_skew 0.030

Activity 3:

- What is Latency

- What is skew

## Specify Routing Rule

- Logically connect PG nets with the pins

- Set the clock routing rules in terms of the Metal layers to be used for clock synthesis, the NDR rules, clocks to be routed.

icc2_shell > set_clock_routing_rules -clocks [all_clocks ] -net_type {internal} -rules clk_rule -min_routing_layer M4 -max_routing_layer M7 icc2_shell > set_clock_routing_rules -clocks [all_clocks ] -net_type {root} -rules clk_rule -min_routing_layer M4 -max_routing_layer M7

# Synthesize and Optimize Clock Tree

Now, we have set the Buffers to be used, Clock routing rules, target latency, target skew — run the Clock_opt command. It is the main core command to synthesize clock tree, routing of clock nets, extraction, optimization and hold time violation fixing of the design.

icc2_shell > clock_opt

Activity 3:

icc2_shell > man clock_opt

Note down the steps involved in this command

## Logically Connect PG Nets with the Pins

- Make the logical connection of PG nets for all the standard cells

icc2_shell > connect_pg_net -net VDD [get_pins -hier * - filter "name == VDD"] icc2_shell > connect_pg_net -net VSS [get_pins -hier * - filter "name == VSS"]

- After clock_opt is executed you can inspect the clock tree and query for the various attributes.

## Inspect the Clock Tree

- View the Clock Tree

- In GUI Select, Highlight → Color By → Clock Tree

Fig: Clock Tree highlighted via Highlight → Color By → Clock Tree

- Reports Constraints

icc2_shell > report_constraints -all_violators

Sample output:

0.66 f 0.66 0.00 -0.00 REG2REG Mode: func Corner: fast Scenario: func_fast max_transition Net Required Actual Transition Transition Slack Violation ---------------------------------------------------------------------- GPRs/gre_net_4719 0.50 0.70 -0.20 (VIOLATED) PIN : GPRs/U366/A2 0.50 0.70 -0.20 (VIOLATED) PIN : GPRs/gre_mt_inst_23108/X 0.50 0.70 -0.20 (VIOLATED) PIN : GPRs/U417/B1 0.50 0.70 -0.20 (VIOLATED) PIN : GPRs/U594/B1 0.50 0.70 -0.20 (VIOLATED) PIN : GPRs/U592/B1 0.50 0.70 -0.20 (VIOLATED) PIN : GPRs/U590/B1 0.50 0.70 -0.20 (VIOLATED) PIN : GPRs/gre_mt_inst_23107/A 0.50 0.70 -0.20 (VIOLATED) PIN : GPRs/U565/B1 0.50 0.70 -0.20 (VIOLATED) PIN : GPRs/U416/B1 0.50 0.70 -0.20 (VIOLATED) PIN : GPRs/U593/B1 0.50 0.70 -0.20 (VIOLATED) PIN : GPRs/U591/B1 0.50 0.70 -0.20 (VIOLATED) PIN : GPRs/U589/B1 0.50 0.70 -0.20 (VIOLATED) PIN : GPRs/U587/B1 0.50 0.70 -0.20 (VIOLATED) PIN : GPRs/U558/B1 0.50 0.70 -0.20 (VIOLATED) GPRs/n818 0.50 0.68 -0.18 (VIOLATED) PIN : GPRs/U356/A2 0.50 0.68 -0.18 (VIOLATED) PIN : GPRs/U1735/X 0.50 0.68 -0.18 (VIOLATED) PIN : GPRs/gre_mt_inst_23088/A 0.50 0.68 -0.18 (VIOLATED) PIN : GPRs/U368/A1 0.50 0.68 -0.18 (VIOLATED)

- Look for Max_Tran and Max_Cap violations in the design

- Report the Clock Tree

icc2_shell > report_clock_tree_options

Sample output:

icc2_shell> report_clock_tree_options *************************************** Report : target skew and latency Design : ChipTop Date : Wed Apr 5 12:04:16 2023 *************************************** ##Target Skew Clock Corner Target ---------------------------------------------------------- clock (mode func) fast 1.05 clock (mode func) slow 1.05 ##Target Latency Clock Corner Target ---------------------------------------------------------- clock (mode func) fast 0.25 ##Max Level Count Clock Count ---------------------------------------------------------- No max level constraints found. ##Root NDR Fanout Limit Clock Fanout limit ---------------------------------------------------------- No Root NDR fanout limit constraints applied. ##Trunk nets specific drivers

icc2_shell > report_clock_qor

Sample output:

================================================================ ==== Summary Reporting for Corner fast ==== ================================================================ ================= Summary Table for Corner fast ================= Clock / Attrs Sinks Levels Clock Clock Clock Max Global Trans Cap Wire Skew Group Repeater Repeater Stdcell Latency Skew DRC DRC Length Count Area Area --------------------------------------------------------------------------------------------- ### Mode: func, Scenario: func_fast clock M,D 723 9 40 14.12 42.14 0.45 0.09 0 0 2535.06 --------------------------------------------------------------------------------------------- All Clocks 723 9 40 14.12 42.14 0.45 0.09 0 0 2535.06 Warning: Please use -largest / -smallest / -all switches with -show_verbose_paths / -show_paths to report the clock paths. (CTS-956) ================================================================ ==== Summary Reporting for Corner slow ==== ================================================================ ================= Summary Table for Corner slow ================= Clock / Attrs Sinks Levels Clock Clock Clock Max Global Trans Cap Wire Skew Group Repeater Repeater Stdcell Latency Skew DRC DRC Length Count Area Area --------------------------------------------------------------------------------------------- ### Mode: func, Scenario: func_slow clock M,D 723 9 40 14.12 42.14 0.42 0.08 0 0 2535.06 --------------------------------------------------------------------------------------------- All Clocks 723 9 40 14.12 42.14 0.42 0.08 0 0 2535.06 Warning: Please use -largest / -smallest / -all switches with -show_verbose_paths / -show_paths to report the clock paths. (CTS-956)

- Report Utilization

icc2_shell > create_utilization_configuration -scope block core_utilization -include {all} icc2_shell > report_utilization -config core_utilization

- Report Quality of Results

icc2_shell > report_qor -summary

Note the WNS, TNS and Number of violating paths :

- Report Timing

icc2_shell > report_timing -delay_type min icc2_shell > report_timing -delay_type max

- Analyze the reports generated.

Activity 4:

Define

- Setup Time :

- Hold Time :

# Saving the Block

icc2_shell > save_block -as cts_done icc2_shell > close_blocks -force icc2_shell > close_lib