# PD Module 8 — Lab Manual

Routing

# Learning Objective

Perform a routability check on a placed design

Set the routing options/constraints for routing

Perform routing

# Learning Outcome

After completing the lab, you will be able to,

- Invoke the Tool

- Open the Library and CTS completed Block

- Check Design - pre route stage

- Set Routing Options

- Set Routing Layers

- Enabling the Crosstalk aware and Delay Calculations

- Global Routing

- Track Assignment

- Detail Routing

- Route_auto

# Task

- Open the directory which was created in the previous lab

Linux > cd chiptop2.0 Linux > cd pd/work

- Invoke the icc2 tool

Linux > icc2_shell -o ../logs/routing.log

- Open the Library

icc2_shell > open_lib ../lib/chiptop

- To see all the blocks present in the library

icc2_shell > list_blocks

- Now, open the cts_done block which was created in the previous lab

icc2_shell > open_block cts_done

- To open the Graphical User Interface (GUI)

icc2_shell > start_gui

- Open one more terminal in the scripts directory, open “routing.tcl” using “vi” or “gvim” editor command from the scripts directory and go through the file contents.

linux > gvim routing.tcl

# Check Design – Pre Route Stage

- Before performing Routing, execute the following commands and analyze the report

- To Check the readiness of design to perform routing

icc2_shell > check_design -checks pre_route_stage

Sample output:

Date : Wed Apr 5 12:12:33 2023 *************************************** Running mega-check 'pre_route_stage': Running atomic-check 'design_mismatch' Running atomic-check 'scan_chain' Running atomic-check 'mv_design' Running atomic-check 'timing' Running atomic-check 'routability' Running atomic-check 'hier_pre_route' *** EMS Message summary *** ------------------------------------------------------------------ Rule Type Count Message ------------------------------------------------------------------ DFT-011 Info 1 The design has no scan chain defined in the scandef. DMM-102 Info 25 Mismatch type %mmtype is detected on object type %objtype at obj... DMM-103 Info 56 Mismatch type %mmtype is detected on object type %objtype at obj... DMM-105 Info 780 Mismatch type %mmtype is detected on object type %objtype at obj... MV-208 Warn 1 Pin(s) '%portStr' of library cell '%module' is(are) missing the ... TCK-001 Warn 292 The reported endpoint '%endpoint' is unconstrained. Reason: '%re... ------------------------------------------------------------------ Total 1155 EMS messages : 0 errors, 293 warnings, 862 info. ------------------------------------------------------------------ *** Non-EMS message summary *** Rule Type Count Message ------------------------------------------------------------------ PVT-030 2 Corner %s: %d process number, %d process label, %d voltage, and... PVT-031 2 %d cells affected for early, %d for late. PVT-034 2 %d port driving_cells affected for early, %d for late. ZRT-026 8 Layer %s pitch %.3f may be too small: wire/via-down %.3f, wire/v... ZRT-027 1 Ignore %d top cell ports with no pins. ZRT-038 11319 Cannot find via def %s for via-region in cell %s. ZRT-044 48 Standard cell pin %s/%s has no valid via regions. ZRT-511 1 Cell contains tie connections which are not connected to real PG. ZRT-703 1 Option route.detail.force_end_on_preferred_grid will be ignored ...

- Check that there is no error reported. There are other known warnings that can be ignored.

- Check for blocked ports, out-of boundary ports and minimum-grid violations for all design objects.

icc2_shell > check_routability

Sample output:

======================================= == Check for overlap of standard cells == ======================================= >>>>>> No overlap of standard cells found ======================================= == Check for min-grid violations == ======================================= >>>>>> No min-grid violations found ======================================= == Check for out-of-boundary ports == ======================================= >>>>>> No out-of-boundary error found ======================================= == Check for blocked ports == ======================================= >>>>>> Port might be blocked by layer constraint >>>>>> Port might be blocked by check port access >>>>>> No blocked ports found

# Custom VIA Creation

- Custom VIA creation with respect to the design requirements to reduce DRC.

- Create Custom VIA of metal layers 1 and 2

icc2_shell > set_attribute [get_layers M1] routing_direction vertical icc2_shell > create_via_def VIA12_Custom_short -cut_layer VIA1 -cut_size {0.03 0.03} -upper_layer M2 -lower_layer M1 -lower_enclosure {0.002 0.025} -upper_enclosure {0.025 0.002} -is_default icc2_shell >set_app_options -name route.common.connect_within_pins_by_layer_name -value {{M1 via_standard_cell_pins}}

- Create Custom VIA of metal layers 2 and 3

icc2_shell > set_attribute [get_layers M2] routing_direction horizontal icc2_shell > create_via_def VIA23_Custom_short -cut_layer VIA2 -cut_size {0.03 0.03} -upper_layer M3 -lower_layer M2 -lower_enclosure {0.002 0.025} -upper_enclosure {0.025 0.002} -is_default icc2_shell > set_app_options -name route.common.connect_within_pins_by_layer_name -value {{M2 via_standard_cell_pins}}

# Set Routing Options

- Now set the routing options before performing routing.

- Routing options for “Track, Global & Detail Routing” could be timing driven and crosstalk driven.

- Run the following commands to set the options.

icc2_shell > set_app_options -name route.detail.timing_driven -value true icc2_shell > set_app_options -name route.track.timing_driven -value true icc2_shell > set_app_options -name route.track.crosstalk_driven -value true icc2_shell > set_app_options -name route.global.timing_driven -value true

# Set Routing Layers

- Define the routing layers that needs to be used for routing

icc2_shell > set_ignored_layers -max_routing_layer M7 - min_routing_layer M2

- Set the options to allow routing below the minimum layer only for pin connections

icc2_shell > set_app_options -name route.common.global_min_layer_mode -value allow_pin_connection

- Set the option to use a layer above the maximum layer as soft. This sets the tool to routing above the maximum layer is discouraged, but not disallowed.

icc2_shell > set_app_options -name route.common.global_max_layer_mode -value soft

# Enabling the Crosstalk Aware and Delay Calculations

icc2_shell > set_app_options -name time.si_enable_analysis -value true icc2_shell > set_app_options -name time.enable_si_timing_windows -value true icc2_shell > set_app_options -name time.enable_ccs_rcv_cap -value true

- The above specifies whether crosstalk delta delay is considered in timing and optimization. When set to false, crosstalk delta delay is not considered. This option is disabled by default.

icc2_shell > save_block -as initial_route

# Perform Routing

## 1. Global Routing

- Use the below command to do global routing

icc2_shell > route_global

## 2. Track Assignment

- To do the track assignment, assigning tracks for the nets, run the below command:

icc2_shell > route_track

## 3. Detail Routing

- Use the below command to do detail routing

icc2_shell> route_detail

- Can also add argument -max_number_iterations. The default value is 40

## Route_auto

- Route_global, route_track and route_detail can be used in one single command i.e. route_auto, it will do the global routing, track assignment and detail routing.

- Note: You need not run this command if you have run the above three commands.

icc2_shell > route_auto icc2_shell > save_block

## Incremental Route

icc2_shell > route_opt icc2_shell > save_block -as route_done

Activity 4:

- SETUP:  WNS_______________ TNS_______________

- HOLD:  WNS_______________ TNS_______________

# Reduce DRC

- The below command helps to reduce DRC

icc2_shell > route_eco -open_driven true -reuse_existing_global_route true -utilize_dangling_wires true icc2_shell > save_block -as route_eco0_done

# Reports

## 1. Report the Check the DRC

icc2_shell > check_routes

Make note of the following:

Activity 4:

- TOTAL VIOLATIONS =

Total Wire Length : ______________________

Total Number of Contacts : ______________________

Total number of wires : ______________________

Total Number of Routed Wires : ______________________

Total Routed Wire Length : ______________________

Total Number of Routed Contacts : ______________________

Total number of nets : ______________________

Total number of open nets : ______________________

## 2. Check LVS

icc2_shell> check_lvs

## 3. QoR Reports

icc2_shell> report_qor -summary

Activity 4:

- SETUP:  WNS_______________ TNS_______________

- HOLD:  WNS_______________ TNS_______________

## 4. All Constraints Reports

icc2_shell> report_constraints -all_violators

## 5. Timing Report – Setup Detail Reports

icc2_shell > report_timing -delay_type max